Ethernet standard and Specification

Tasks

1.Draw a simple diagram showing the pin layout of an RJ-45 connector and label the color codes for the T568A and T568B Ethernet wiring standards.

Ans :

1. T568A Standard Color Sequence

- Pin 1: White / Green

- Pin 2: Green

- Pin 3: White / Orange

- Pin 4: Blue

- Pin 5: White / Blue

- Pin 6: Orange

- Pin 7: White / Brown

- Pin 8: Brown

2. T568B Standard Color Sequence

- Pin 1: White / Orange

- Pin 2: Orange

- Pin 3: White / Green

- Pin 4: Blue

- Pin 5: White / Blue

- Pin 6: Green

- Pin 7: White / Brown

- Pin 8: Brown

2.Create a table comparing the key specifications of Ethernet standards 10BASE-T, 100BASE-TX, and 1000BASE-T, including speed, cable type, maximum cable length, and connector type.

Ans :

Comparison of Ethernet Standards (10BASE-T, 100BASE-TX, and 1000BASE-T)

| Ethernet Standard | Speed (Bandwidth) | Cable Type | Maximum Cable Length | Connector Type |
| --- | --- | --- | --- | --- |
| 10BASE-T | 10 Mbps | Cat 3 (or better) Twisted-Pair | 100 meters (328 ft) | RJ-45 |
| 100BASE-TX | 100 Mbps (Fast Ethernet) | Cat 5 (or better) Twisted-Pair | 100 meters (328 ft) | RJ-45 |
| 1000BASE-T | 1000 Mbps / 1 Gbps (Gigabit Ethernet) | Cat 5e / Cat 6 Twisted-Pair | 100 meters (328 ft) | RJ-45 |

3.Given a scenario where you need to connect two laptops directly using Ethernet, explain which cable (straight-through or crossover) you would use and why.<br><br><em><strong>Hint:</strong> Think about how transmit and receive pairs are wired in each cable type.</em>

Ans :

if I need to connect two laptops directly using Ethernet, I would use a crossover cable.

Why: A crossover cable swaps the transmit (TX) and receive (RX) wire pairs, allowing both laptops to communicate directly without a router or switch.

4.List three real-world devices in your home or college that use Ethernet connections, and for each, specify the likely Ethernet standard (e.g., 100BASE-TX, 1000BASE-T) they support.

Ans: Real-World Devices Using Ethernet Connections

- Smart TV / Android TV Box

  - Likely Standard: 100BASE-TX or 1000BASE-T (Gigabit Ethernet)

  - Usage: Connected via an Ethernet cable to stream high-definition 4K video content smoothly without Wi-Fi interference.

- Wi-Fi Router / Network Switch

  - Likely Standard: 1000BASE-T (Gigabit Ethernet)

  - Usage: Local area network (LAN) ports on home routers or college switches use high-speed Gigabit ports to distribute internet traffic to multiple wired clients.

- Desktop PC / College Lab Computer

  - Likely Standard: 1000BASE-T (Gigabit Ethernet)

  - Usage: Motherboards feature built-in Gigabit Ethernet network interface cards (NICs) to handle heavy data transfers and online lab work.