IP Addressing And Subnetting

Tasks

1.List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

Ans:

| IP Class | Purpose / Size | Real-World Example |
| --- | --- | --- |
| Class A | For massive networks with millions of devices. | Global Tech Giants: Mega-corporations like Apple or IBM own Class A blocks to manage their vast global networks. |
| Class B | For medium to large networks. | Universities & Large Offices: A large university campus uses Class B to assign IP addresses to thousands of student and staff devices. |
| Class C | For small networks. | Home & Cafe Wi-Fi: Your home Wi-Fi router uses Class C (like 192.168.1.X) to connect your phone, laptop, and smart TV. |
| Class D | For Multicasting (sending data to many devices at once). | Live Sports Streaming: Services like Hotstar or JioCinema use Class D to broadcast live cricket matches to millions of screens simultaneously. |
| Class E | Reserved for experimental and research work. | R&D and Military: Organizations like NASA or internet research bodies use Class E to test new networking technologies. |

2.Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses.<br><br><em><strong>Hint:</strong> Show your working steps for each calculation.</em>

Ans :

1. Identify the Basics

- IP Address: 192.168.10.45

- Subnet Mask: 255.255.255.0

Because the subnet mask has 255 in the first three parts (octets), it means the Network portion is fixed to the first three numbers (192.168.10), and only the last number (45) belongs to the Host portion.

Step-by-Step Calculations

- Step 1: Network Address

  - How to find it: Keep the network part exactly the same and change the host part (the last number) to 0.

  - Calculation: 192.168.10. + 0

  - Answer: 192.168.10.0

- Step 2: Broadcast Address

  - How to find it: Keep the network part exactly the same and change the host part (the last number) to the maximum possible value, which is 255.

  - Calculation: 192.168.10. + 255

  - Answer: 192.168.10.255

- Step 3: Range of Valid Host Addresses

  - How to find it: Valid hosts are the usable IP addresses that sit right between the Network Address and the Broadcast Address.

  - First usable IP: Network Address + 1 \(\rightarrow \) 192.168.10.1

  - Last usable IP: Broadcast Address - 1 \(\rightarrow \) 192.168.10.254

  - Answer: 192.168.10.1 to 192.168.10.254

3.Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

Ans :

The Subnet Calculation

- Base Network: 10.0.0.0/24 (provides 256 total IP addresses).

- Requirement: 4 subnets.

- The Math: To get 4 subnets, we steal 2 bits from the host portion (2² = 4). This changes our subnet mask from /24 to /26 (or 255.255.255.192).

- Block Size: Each subnet will have exactly 64 total IP addresses (62 usable for devices).

The Subnetting Plan

| Department / Group | Network Address | Subnet Mask (Subnet Prefix) | Usable Host IP Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1. Delivery Partners | 10.0.0.0 | 255.255.255.192 (/26) | 10.0.0.1 to 10.0.0.62 | 10.0.0.63 |
| 2. Restaurants | 10.0.0.64 | 255.255.255.192 (/26) | 10.0.0.65 to 10.0.0.126 | 10.0.0.127 |
| 3. Customers | 10.0.0.128 | 255.255.255.192 (/26) | 10.0.0.129 to 10.0.0.190 | 10.0.0.191 |
| 4. Admins | 10.0.0.192 | 255.255.255.192 (/26) | 10.0.0.193 to 10.0.0.254 | 10.0.0.255 |

Why this plan works well for the startup:

- Security: Admins are safely isolated from customers and delivery riders on their own separate network.

- Efficiency: No IP addresses are wasted, and each group has plenty of room (62 slots) to connect local office test devices, local servers, or network equipment.

4.You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet.<br><br><em><strong>Constraint:</strong> Show your calculation steps and final answers.</em>

Ans :

1. Analyze the Given Information

- Base Network IP: 172.16.0.0

- Default Subnet Mask: /16 (which is 255.255.0.0)

- Required Subnets: At least 20

2. Calculation Steps

Step 1: Calculate the Number of Subnet Bits to Borrow

To find out how many host bits we need to borrow to create at least 20 subnets, we use the formula: \(2^n \ge \text{Required Subnets}\) (where \(n\) is the number of borrowed bits).

- Try 4 bits: \(2^4 = 16\) subnets (Too small, we need at least 20)

- Try 5 bits: \(2^5 = 32\) subnets (This works! 32 is greater than or equal to 20)

So, the network administrator must borrow 5 bits from the host portion.

Step 2: Calculate the New Subnet Mask

The default mask is /16. We add the 5 borrowed bits to it:

- New CIDR notation: \(16 + 5 =\) /21

To convert /21 into a decimal subnet mask:

- The first two octets are completely full: 255.255

- The third octet gets 5 network bits and 3 host bits: 11111000 in binary, which equals 248 in decimal.

- The fourth octet is empty: 0

- New Subnet Mask: 255.255.248.0

Step 3: Calculate the Number of Available Hosts Per Subnet

A standard IP address has 32 bits total.

- Remaining bits for hosts: \(32 \text{ total bits} - 21 \text{ network bits} = 11 \text{ host bits}\)

Now, we use the host formula: \(\text{Usable Hosts} = 2^h - 2\) (where \(h\) is the number of host bits, and we subtract 2 for the Network and Broadcast addresses).

- \(2^{11} - 2 = 2048 - 2 =\) 2,046 hosts per subnet.

5.A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

Ans :

The reason your device isn't connecting is because 192.168.1.300 is an invalid IP address.

The Error

An IP address is made up of four numbers separated by dots, and each individual number can only go from 0 to 255. Because the last number in your IP is 300 (which is higher than 255), your network router completely rejects it as a glitch or a typo.

What a Valid IP Looks Like

Since your home network uses the 192.168.1.X subnet, a valid IP address for your device must keep the first three parts exactly the same, but change the last number to anything between 1 and 254.

For example, any of these would work perfectly:

- 192.168.1.30

- 192.168.1.45

- 192.168.1.100

How to fix it: Go into your device's network settings and change the IP configuration from "Manual" to "Automatic (DHCP)". This will let your router automatically give your device a valid, working IP address!